# Oklahoma City plant

- Built in 1979

- 1 of 28 GM plants in US

- 1 of 2 GM plants assembling type A car

- 3rd/4th best plant in North America in previous 2 years (rated by JD Power)

- 3 million square feet

- 5300 employees (~4900 line workers)

- 2 production shifts

- Assembles 2 nameplates of “body type A car” (Cutlass Ciera and Buick Century)

# Car Assembly

- Classic example of moving assembly line

- “Line” is actually combination of 2 subassemblies (body and engine cradle)

- 1800 cars on line at once

- 28.5 hours/car

- 50 seconds/car/work station

- 2 workers/station (average), 1 on each side

- “Overcycled” jobs (taking longer than average station rate) must be balanced by undercycled jobs

- Overcycling predominant on lines 1 and 2 where options added

- Workers’ instructions for what components to add found on “broadcast”/”manifest” sheets on line

# Planning Production

Essential nature of scheduling

- Inventory control is essential due to bulk and expense of parts

- No finished goods inventory is maintained, all shipped promptly

- As such, all cars for particular destinations must have similar completion schedules

- Complicated because all cars are produced to order (make, model, color and options)

“Synchronous manufacturing” (just-in-time manufacturing) planning

- Corporate office (Detroit) sets rough production schedule for year

- C-P-C central office (Warren, MI) sets plant production to met customer “target build” date

- “Stable schedule process” sets firm 10-15 day car-by-car production schedule

  - “Auto sequence” computer program for handling “restrictions”creates a “production point of use” plan for a “stable schedule process”

    - restriction: color sequence (white, lt. blue, dk. blue, blue, red, brown, silver) due to cleaning difficulties

    - restriction: only 54 gates for holding side panels, limiting the number of any one model that can be built in a sequence

    - restriction: length of power door/window installation

  - “Point of use” plan  transmitted electronically to suppliers each Monday, along with 20-week planning schedule (3-week “stable schedule” and 17-week forecast) (future plan for one merged document)

  - Transparency and stable demand of “auto sequenced” production decreased supplier costs

- 98% of “target build” production is completed on schedule

- Reduced expediting and improved quality (at plant and supplier)

# Purchasing and Raw Materials Inventory/Control

- 4,046 parts and 493 suppliers; reduction from 7059 parts/783 suppliers (when four nameplates were manufactured)

- C-P-C central office negotiates all long-term contracts

- Only 1-3 days’ worth of raw materials inventory held; <1 day for more expensive items

- Most expensive parts (350) = 80% total materials inventory cost

- Inventory levels maintained dependent on

  - Volume of use (inverse relationship)

  - Value (inverse)

  - Size (inverse)

  - Transportation distance (direct relationship)

- Monitoring of supplies by materials department and line workers/supes

- Inventory turns rising from 30/40 (1981) to 56 (1991)

# Synchronous Manufacturing (transition to)

- To eliminate non-value-added activities (unnecessary activities/controls, excess inventories, and working space)

- C-P-C plant is less automated than other “high tech” plants (30/40 robots vs. 150)

- Existence of plant “support network” for generating, analyzing and implementing improvements

- “Pull card” system to time release of materials used during production

- Only 4 hours of parts on line

- Supes have “cradle to grave” responsibility for process segments (increasing responsibility)

- “Design for Assembly” system to learn from workers how to make assembly easier/higher quality

- No layoffs in transition to synchronous manufacturing; worker reassignment to other tasks

- Gains in productivity, space saved, inventory dollars saved, and quality

- Preceded by thirteen 4-day workshops by plant leaders in 16 months

# Revising and controlling operation

- Little plant autonomy in materials purchase or production mix (corporate or C-P-C central office); but much autonomy in line design and management

- Quality control is essential to synchronous manufacturing

  - No inspection of supplies on delivery

  - “Build-in-Station” system for line workers paid more to assume responsibility of inspection

- “Matching the voice of the customer to the voice of the process” philosophy to introduce improvements into the line

- QC Department “reliability group” responsible for safety, supplier quality, engineering change orders

- QC Department “audit group” responsible for grading production quality of shifts (18 cars/shift, 4 complete, 14 most common problems); 3 cars/month comprehensive eval along with competitor car

- 10 times/year random audit from corporate headquarters of 20 cars

- Statistical data collected for implementing preventative measures

- Employee ability to stop entire line if problem encountered

- Employee visits to suppliers to discuss specific requirements

# Industrial engineering

Responsible for

- Translating design into procedure

- Laying out line

- Assigning work to stations

- Establishing level of work and monitoring

- Designing improvements

# Supervision

- General supervisor: 4-5 supervisors

- Supervisor (quality check and trouble-shooting): 30-35 workers

- Support person: 7-10 person unit

- “Volutnary Input Process” (VIP) – worker teams (of 6-10) met on Wednesdays to discuss potential improvements; 75-80% plant participation

## DISCUSSION

# Flow

- Machine-line process flows like CPC are some of the most well-defined but complex flows

- Complex information flow also (mostly top-down, but materials tracking in all directions, inside and outside plant)

# Capacity

- Relatively unambiguous: 100% when on, 0% when off

- Total capacity utilization = time on/total possible time (planned shifts)

- Short-run capacity severely constrained; modifiable only by scheduling

- Medium-run modification can be substantial w/o new PPE by re-balancing line

# Workforce demands

- “slaves to the iron monster”

- CPC plant issues:

  - Personnel department put great emphasis on worker training/safety/involvement

  - Supervisors recognized achievement and assisted workers with work/non-work issues

  - Engineers eased/eliminated difficult/disagreeable tasks

# Management demands

- Machine-paced line process: Price competition, performance, reliability

- Cost reduction measures:

- Balance

- Materials management

- Technological change

- Capacity planning

- Workmanship measures:

- Product design

- Workforce management