Convergence of

Circuit Switching and Packet Switching

in Future Communications Networks

By: Kevin DeMartino

Independent Consultant

kevindemar@verizon.net

# Abstract

A networking approach is proposed that combines the advantages of circuit switching and packet switching in an integrated communications network. With this approach, control signals are separated from the data and transmitted separately in a common control channel. Control and signaling operation can be viewed as a logical extension of generalized multi-protocol label switching (GMPLS). A variable number of data channels are dynamically assigned to each connection through the network. Channels are quickly switched in to accommodate the data flow through a connection and switched out when they are no longer needed. With this dynamic channel switching approach, a continuous data stream is not altered as it transverses the network and data channels are efficiently utilized even if the data source is bursty. The proposed approach is compatible with legacy networks and can provide a mechanism for convergence of circuit switching and packet switching. Most of the benefits of the proposed approach can be achieved if only network and server equipment, but not client equipment, is upgraded to support dynamic channel switching.

## The Goal of an Integrated Communications Network

In the past, a communications network was usually designed to provide a specific communication service. Networks designed around a specific service include: the telephone networks, which were designed to support voice over circuit switched connections; the cable television (TV) networks, which were designed to provide broadcast video; and the Internet, which was designed to support computer communications using packet switching. Subscribers needed to access multiple networks in order to obtain a full range of communication services.

For both clients and servers, access to the Internet has usually been provided by circuit switched connections through the telephone networks. Typically, a client is connected to an Internet service provider (ISP) via a dial-up connection, which usually involves a local (non-toll) call over a relatively short distance. For servers, both dial-up and permanent (leased line) connections to ISPs over relatively short distances are common. Data is packet switched as it traverses the Internet from the client’s ISP to the server’s ISP and vice versa. Very long distances can be covered by data flows through the Internet, with most of the data flowing in the direction from the server to the client, which is usually referred to as the downstream direction.

It is desirable to have a common, integrated network that can provide each subscriber with a full range of communication services over a single broadband access line. The capacity provided to each subscriber should be sufficient to accommodate full motion video among other high data rate communication functions. High speed network access can be provided over twisted pair telephone lines using digital subscriber line (DSL) techniques or over TV cable using hybrid fiber-coax techniques. The current DSL and HFC (cable modem) implementations provide high speed access to Internet servers, which can supply audio and video streams in addition to supporting traditional computer data communications, i.e., file transfers. With these approaches, integrated services can be supported, however, the supporting networks are not fully integrated. The various networks, including the DSL and HFC access networks, the circuit-switching voice networks, the broadcast video networks, and the packet-switching Internet,  interface with each other. However, these networks maintain distinct identities. The integration of network services is hampered by the fact that some services, such as voice, are currently based on circuit switching, while other services, such as file transfers, are currently based on packet switching.

With integrated services digital networks (ISDN), an attempt was made to establish integrated networks that could support both voice and data communication services. Narrowband ISDN [1] is primarily based on circuit switching with 64 Kb/s channels assigned to particular connections. The subscriber data rates that can be supported by narrowband ISDN interfaces are  relatively low, and consequently, the video capabilities of ISDN are very limited. Broadband ISDN (B-ISDN) [2] was designed to provide data rates that could easily support the full range of services, including full motion video. B-ISDN is based on a  form of packet switching called Asynchronous Transfer Mode (ATM). ATM was designed to transport all types of data (e.g., voice, video, or computer files) using small, fixed size cells.

B-ISDN with ATM has not lived up to its promise and has not been generally accepted as solution to the integrated services problem. Instead, another approach, which leverages the success of the Internet, is based on packet switching and the Internet Protocol (IP). This IP-centric approach has gained widespread acceptance. However, in order to achieve the network performance needed for integrated services, additional protocols beyond IP are required. The leading approach for providing the required performance is based on multi-protocol label switching (MPLS)[3]. MLSP moves the integrated network away from the connectionless paradigm of IP and make the integrated services network not quite IP-centric.

#### Convergence of Circuit Switching and Packet Switching

An integrated services network can be based on circuit switching, as used in narrowband ISDN, or on packet switching, as used in B-ISDN or IP-centric networks. Another alternative approach, which is the approach followed here, would involve the convergence of circuit switching and packet switching techniques.

Circuit switching networks, such as the telephone networks, establish end-to-end physical layer connections through the network. Transmission resources, i.e., channels, are dedicated to a particular connection for the duration of a call. With circuit switching, delays are minimized and fixed. A stream of data can be transported through a circuit switching network without suffering any distortion. Also, the processing performed by a circuit switching network is relatively simple and straightforward. On the other hand, the utilization of dedicated channels can be low if there is a significant variation in the data rate. Consequently, circuit switching networks are inefficient for transporting bursty data.

With packet switching, transmission resources are dynamically shared based on demand. This makes packet switching efficient for transporting bursty data, such as the data flows associated with file transfers. However, packet switching introduces randomly varying delays into the data stream, which can be problematic for the transmission of continuous data, such as voice and video data streams. Also with packet switching, decisions with respect to switching (or forwarding) of packets must be made on the fly based on information contained in the packet header. This complicates data processing in the network.

The Internet Engineering Task Force (IETF) is developing network control and signaling standards based on MPLS that apply to both circuit switching and packet switching. This set of standards, which fall under the collective heading of generalized MPLS (GMPLS)[4], move networks away from connectionless packet switching to a more connection-oriented operation and foreshadow the convergence of circuit switching and packet switching.

This paper proposes a network architecture based for an integrated services network that is based on dynamic switching of the channels assigned to connections through the network. This network combines the advantages of circuit switching and packet switching. Channels are quickly assigned to a connection to support a particular data flow and quickly released when they are no longer needed. Network control and signaling are based on a logical extension of the GMPLS concept. With the proposed approach, bursty data can be efficiently supported while continuous data streams are transported through the network without distortion.

The proposed network is compatible with legacy telephone networks and with the Internet. To take full advantage of this network’s capabilities, client and server equipment must be upgraded to incorporate a dynamic channel switching network interface. However, most of the benefit of dynamic channel switching can be realized if only Internet servers are upgraded and clients continue to operate as they currently do. Consequently, the proposed approach provides a mechanism for facilitating the convergence of circuit switching and packet switching.

Operation of Circuit Switching Networks

Circuit switching techniques have been employed by telephone networks for over 100 years.[5] Figure 1 illustrates the operation of a circuit switching network. End-to-end physical layer connections are established between subscriber nodes, which include client nodes, server nodes, and nodes that combine both client and server functions. For voice operation, a connection is established between peer nodes when one subscriber “calls” another. Data transfers usually involve a connection between a client and a server, with most of the data flowing downstream from the server to the client.

Each subscriber node has a permanent, dedicated connection to the network, which usually consists of a pair of copper wires that are twisted together. In the old days a connection was established between  subscriber lines by physically linking wires together through a series of patch panels or electro-mechanical switches so that a closed circuit was formed with electric current flowing from a subscriber on one end through the network to a subscriber on the other end, and back again. In today’s networks, the subscriber line on each end is connected to a network channel and channels are linked together through the network to form a continuous end-to-end channel, or connection. Although the process of  establishing this end-to-end connection is still called “circuit switching”, “channel switching” would be a more accurate description of what actually takes place within the network. Channels forming the end-to-end connection are dedicated for the duration of the call.

To cause a connection to be set up or released, a subscriber must signal the network.[6] Similarly, control signals must be transmitted within the network to cause channels to be assigned to a particular connection. Control signals can be associated with a particular data channel and transmitted in the channel along with the data. Alternately, a common signaling channel can be used to control multiple data channels. Channel-associated signaling is employed when a subscriber dials the telephone to provide the network with the information (the telephone number) it needs to establish a connection. In this case, signaling information is sent from the subscriber to the network in the same band (or channel) used for voice transmission. Within telephone networks, control information is usually transferred between  nodes over a common signaling channel using the Signaling System Number 7 (SS7) set of protocols.[7] Network control signals may follow different paths than the data they are controlling.

Figure 1 – Circuit Switched Operation

With the advent of ISDN, common channel signaling was extended to the user-to-network interface (UNI).[8] ISDN provides the user with a separate signaling channel, the D channel, which is used to control the assignment of data transfer channels (B channels) to a particular connection. Using the D channel signaling, the subscriber can request that the network allocate additional B channels to an active connection. Unfortunately, most narrowband ISDN interfaces provide only two B channels to a subscriber, which greatly limits the utility of narrowband ISDN. With B-ISDN, data rates and the number of equivalent data channels are greatly increased. However, B-ISDN inexplicably rejected common channel signaling in favor of ATM.[9]

Data is usually transferred digitally through circuit switching networks. With the telephone networks, analog data is usually converted to digital form at the local central office (CO). In the case of  ISDN, and other digital lines, data is already in digital form at the subscriber interface.

Delays through circuit switching networks are minimal and fixed. There may be some small buffering delays, but except for local traffic, these are usually small compared to unavoidable propagation delays. If the bit error rate is low, which is normally the case, then data will be transported through a circuit switching network without distortion. This situation is illustrated by Figure 2, where the data stream exiting the network is identical to the data stream entering the network except for a small fixed delay. This property of the network is important for continuous data streams, such as those produced by voice and video sources, which can be adversely affected by variable delays.

Figure 2 – Undistorted Data Flow Through a Circuit Switching Network

Quality of service (QOS) measures include data rates, delays, delay variations, and error rates. A circuit switching networks can provide a guaranteed QOS with a fixed data rate, a low fixed delay, and without errors due to lost or out of order data. These characteristics make circuit switching operation compatible with legacy networks and most data sources.

Compared to packet switching, circuit switching is relatively simple and straightforward to implement. Circuit switched channels, such as Synchronous Optical Network (SONET) or Synchronous Digital Hierarchy (SDH) channels, can be readily aggregated, which makes circuit switching networks scalable. If the data rates are high enough, it would be practical to implement circuit switching with optical components, which is currently not an option for packet switching.

The primary disadvantage of circuit switching is that it is inefficient for transporting bursty data. Channels with sufficient capacity to handle the required peak data rate for a particular connection must be dedicated to that connection. If the data rate is bursty, i.e., the peak data rate is much greater than the average data rate, then the channels assigned to a connection will be underutilized most of the time. This is particularly true for human initiated file transfers, which tend to be very intermittent and where the utilization may be a few percent or less. For applications involving continuous data flows, such as voice or streaming audio/video, the percentage variation in the data rate is much less than the percentage variation for file transfers. Consequently, circuit switching is much more efficient for continuous data sources than it is for intermittent sources.

Operation of Packet Switching Networks

Packet switching operation involves breaking up data blocks into smaller blocks, or packets, which contain header and trailers in addition to the data. Packets are switched through the network based on information provided in the packet headers. With packet switching, transmission resources (channels) are dynamically  shared by multiple data sources rather than being dedicated to a single source. This makes packet switching more efficient than circuit switching for transporting bursty data. A broad definition of packet switching would include networks based on the Transmission Control Protocol (TCP)/Internet Protocol (IP), X.25, Frame Relay, Ethernet, and ATM.

The Internet  can be viewed as a large number of packet switching networks interconnected by routers.[10] These routers use IP as a common protocol for transferring packets through the Internet. Within the subnets that constitute the Internet, a diverse set of  protocols, including X.25, Frame Relay, or ATM, are used for transferring packets.

Figure 3 illustrates how clients and servers are connected to the Internet through the telephone networks. Clients typically establish dial-up circuit switched connections with Internet service providers (ISPs).  Servers may be connected to the Internet by dial-up lines (as shown in Figure 3) or by leased lines, or in some cases, servers may be co-located with ISPs.

The IP router at an ISP’s point of presence (POP) can be considered to be the point of entry to the Internet. Within the Internet, routers are usually connected to each other by leased lines contained within  trunks. If these leased lines are viewed as part of the telephone networks , then data flowing through the Internet would flow into and out of the telephone networks as illustrated by Figure 3. Even within the individual packet switching networks of the Internet, resources leased from the telephone networks are often used to establish physical connections. To simplify the figure, these connections are shown as remaining within the Internet. From the perspective of Figure 3, the Internet is viewed as being implemented on top of the telephone networks with the Internet being highly dependent on the telephone networks.

The primary rationale for packet switching networks, in general, and the Internet, in particular, is that they do not require dedicated end-to-end connections between clients and servers. For the most part, physical connections would be shared rather than dedicated. In Figure 3,  the physical connection from the client to the Internet (ISP POP), which typically would cover a distance of several miles, would usually be dedicated to the flow of data between a single client and a single server. Physical connections through the Internet, which could cover a distance of thousands of miles, would be shared by many client/server data flows. For most servers, the physical connection between the server and the Internet would be shared could be shared by multiple data flows. Thus, packet switching provides a mean for efficiently utilizing the existing telephone infrastructure without changing it.

The TCP/IP approach to packet switching, upon which the Internet is based, offers some particular advantages compared to alternative packet switching approaches. IP is a connectionless protocol with globally significant addresses contained in the header of IP packets. This means that IP packets can be treated independently of each other. Much of the TCP/IP functionality , such as packet sequencing, flow control, and error control, has been assigned to TCP. With TCP, end-to-end virtual connections, which set parameters for transferring data without assigning physical network channels, are established between subscribers. With this type of operation, TCP is implemented in the end stations, but not seen by the network itself. This allocation of  functions simplifies processing within the network and facilitates interfacing between heterogeneous networks. The widespread acceptance of TCP/IP, compared to other packet switching protocols such as X.25, and the success of the Internet can attributed in part to these advantages.

Figure 3 - Relationship Between Telephone Networks and the Internet

The global addresses in the IP header provide the information required to route packets through the Internet. The term “routing” usually refers to determining the route that a packet should follow in traversing the network. The term “forwarding” refers to moving a packet from a particular input port of the router (or more generally, the packet switch) to a particular output port. Routing requires the exchange of information among Internet routers using protocols such as the Border Gateway Protocol (BPG) and the Open Shortest Path First (OSPF) protocol.[11] This exchange, which takes place in the control plane as opposed to the data plane, is analogous to common channel signaling in circuit switching networks. Forwarding requires processing of information in the packet header, which is linked to data in the packet payload. This linking is analogous to channel-associated signaling in circuit switching networks.

Problems associated with packet switching networks stem largely from the fact that packet switching causes an inevitable distortion of the stream of data flowing from a data source. With packet switching, data from a source is broken up into packets and headers and trailers are attached to these packets. Of course, the data can be reassembled to its original form at the receiving end if uncorrectable errors are not introduced in the transmission process. Consequently, for data transfers that are not time sensitive, such as most file transfers, en route distortions associated with packet switching are not usually a problem. However, for continuous data sources, such as voice or streaming audio/video sources, distortions introduced by packet switching can be problematic. For example, consider the data stream shown in Figure 4. In this case, the data stream entering the packet switching network has a uniform spacing between data blocks. A packet switching network can introduce randomly varying delays, which would cause the data blocks to arrive at the receiver with a variable spacing and possibly out of order. If the random delay variations are too large, then the data may arrive too late to be used by a time sensitive application or too much data may arrive too early to be accommodated by a finite buffer.

Figure 4 – Distortion of Data Stream Through a Packet Switching Network

Even if the bit error rate is close to zero, the distortions of the data stream described above can produce errors in the received data. Specifically, these errors are the result of lost or out of order packets, discarded late packets, and buffer overflows. For time insensitive applications, these errors can be corrected by the error control features of higher layer protocols, such as TCP. However, these errors can adversely affect time sensitive applications, particularly those applications associated with continuous data sources.

In addition to problems associated with errors, packet switching introduces incompatibilities with existing networks, particularly legacy telephone networks. Generally, telephone networks receive and deliver data at fixed rates, while packet switching networks generally receive and deliver data at varying rates. These incompatibilities can be alleviated by buffering the data flow between networks, which introduces delays.

In packet switching networks forwarding decisions must be made on-the-fly. This makes the processing required for packet switching more complex and demanding than the corresponding processing for circuit switching.

In addition to the generic packet switching problems described above, there are problems specifically related to the TCP/IP approach. TCP is implemented in the end stations and is not seen by the network. This means that functions that strongly impact network performance, such as virtual connection establishment (and release) and flow control, are outside the control of the network. IP, which is a connectionless protocol and has very limited functionality, is the operative protocol for network elements, specifically network routers. Because of the limitations of IP, it is difficult for a network  relying solely on IP to control congestion and to provide a guaranteed quality of service.

Various means have been suggested for dealing with the packet switching problems described above, particularly the problems associated with TCP/IP. One approach is to provide a connection oriented capability through the control plan using Multiprotocol Label Switching (MPLS).[3] MPLS provides a means for establishing virtual connections, which are called label switched paths (LSPs), through the exchange of control messages among network routers. MPLS uses a label distribution protocol (LDP), which is a set of procedures by which network elements inform each other of the meaning of labels used to forward traffic. Transmission resources can be assigned to particular LSPs by exchanging reservation messages among the routers using the Resource Reservation Protocol (RSVP) .[12] MPLS also defines generalized labels to be attached to IP packets (and other types of packets). These labels identify that a packet belongs to a particular LSP  requiring a certain category of service and can be used by routers to forward packets without relying on the full IP addresses. With MPLS and supporting protocols, it becomes more practical to provide a certain quality of service compared to what can be achieved using IP alone.

The IETF is in the process of extending MPLS so that it can be used in a manner similar to circuit switching, and in particular, the switching of optical channels. Generalized MPLS (GMPLS) provides for implicit labels, which can be used to associate a data flow with a particular channel. With GMPLS, the control plane is separate from the data plane so that control signals and data are in different channels and can be processed separately. This type of operation is similar to common channel signaling approach used in conjunction with narrowband ISDN. One option for GMPLS is to use RSVP with traffic engineering extensions (RSVP-TE) as the label distribution protocol.[13]

Rationale for Dynamic Channel Switching

If there were no legacy networks and if a communications network were to be built from scratch, then it would not make sense to design separate networks, with different protocols, for voice, video, and data communications. Instead, it would make sense to design an integrated network that would provide a full range of communication services using common protocols and similar modes of operation.

The previous two sections emphasized the significant differences between circuit switching and packet switching. However, circuit switching and packet switching also have certain things in common. This section emphasizes this commonality and describes how an ideal network would combine the features of both circuit switching and packet switching.

With both circuit switching and packet switching, signals are switched in both space and time. Figure 5 shows a common switching element that can be used for circuit switching and/or packet switching. Each of the input lines (or space channels) is broken up into time intervals (or time channels). If an input signal contains multiple carrier frequencies or wavelengths, the signal would be converted to signals on multiple lines prior to switching. Thus, a single fiber containing multiple channels would be equivalent to multiple fibers each containing a single wavelength channel. The switching element of Figure 5 maps space-time channels at its input onto space time channels at its output.

Data on each of the N input lines would be stored for a short time interval in the input buffer. Data can be read out of the input buffer in an order different from the order in which it was read in, which would allow time slots to be interchanged. Data is switched from the input lines to the output lines of the N by N space switch, with the space switching pattern possibly changing from time slot to time slot. Data exiting the space switch is stored in the output buffer while it is waiting to be placed on an output line. In reading out the data, the output buffer can perform a second interchange of time slots. Thus the switch employs three stages of switching, time switching, space switching and time switching again. With this arrangement of switching elements, blocking within the switch can be eliminated.[14] Thus, data in any time slot on any input line can be placed in any time slot on any output line. Of course, there would be a conflict if the switch attempted to map data from two or more inputs onto the same space-time channel at the output.

### Figure 5 – Common Switching Element

With circuit switching operation, the time slot interchange and space switching functions within the switch of Figure 5 would be controlled based on information provided in the signaling channels. The controller would monitor the signaling channels and would determine the switching pattern for mapping input space-time channels onto output space-time channels. This pattern would include time slot interchange parameters for all input and output lines and space switching parameters for all time slots. With circuit switching, the switching pattern is  predetermined for each connection through the switch. The switching pattern is modified whenever a connection is established or released or when the number of channels assigned to a connection is varied. The controller would send updated interchange and switching parameters to the buffers and the space switch, respectively, whenever the switching pattern is modified.

Packet switching operation of the switch of Figure 5 would be similar, but more complex. As with circuit switching operation, time slot interchange and space switching patterns are determined by the controller. Unlike the circuit switching case, the switching pattern is not predetermined for packet switching operation. Instead the switching pattern must be determined on-the-fly based on information contained in the packet headers. The controller must read the header of each packet entering the switch. If the header indicates that the packet is part of an existing data flow, the routing through the network should already have been determined and the output line should be available in an existing table. Otherwise the routing of the packet will have to be determined by the routing algorithm, which may involve the exchange of control messages with other switches. Once the output line for the packet is determined, the controller must determine when the packet should be read out of the input and output buffers. Also, the packet header may need to be modified as the packet passes through the switch. The on-the-fly processing described above make the latency and throughput requirements on the controller considerably more stringent for packet switching compared to the corresponding requirements for circuit switching.

The packet switching operation described in the previous sections and the operation of the common switching element described above can be improved upon. Normally, the information required to switch or forward a packet is contained in the packet header, which is attached to the data block, as shown in Figure 6A. This means that switching/forwarding decisions must be made on-the-fly, which imposes strict latency requirements on the switch of  Figure 5. Suppose the headers are sent in advance of the data, as illustrated by Figure 6B. This would allow the switch to determine the switching pattern in advance of receiving the data, which would allow the latency requirements to be relaxed. It would also allow the switch to initiate flow control before the actual onset of congestion.

### Figure 6 - Rationale for Dynamic Channel Switching

Next suppose that instead of sending a separate header for each data block (or packet), multiple headers are combined into a single header, as illustrated by Figure 6C. This would be particularly advantageous for the case where multiple successive packets are part of the same data flow and are to be sent to the same destination. Also, combining of headers would be advantageous for a sequence of packets that are generated at regular intervals. If multiple headers are combined, the controller could determine the switching patterns for multiple packets at the same time. This would significantly reduce the computational burden, and the throughput requirement on the controller could be significantly relaxed.

Finally, suppose that multiple data blocks are combined into a single data block, as illustrated by Figure 6D. At this point, it would no longer be necessary to break source data into packets. Consequently, it would be possible to directly transmit data as it flowed from the data source. If the bit error rate is low enough and sufficient network capacity is provided, then the data stream delivered by the network to the receiver would be the same as the source data stream, except for a small fixed delay.

Operation of an Ideal Network

Figure 7 shows the interconnection of multiple switching elements of the type shown in Figure 5  to form a dynamic channel switching network. Each node (switching element) is connected by transmission lines to two or more other nodes. The solid lines in Figure 7 indicate connections carrying data, while the dotted lines indicate connections carrying control signals. Control signals and data would flow through the same transmission lines, which in this case are fiber optic cables. However, control signals would be assigned channels that are separate from the data channels. All these connections would be full duplex, i.e., they carry data/signals in both directions. However, the capacities of the channels assigned to the two directions may be different. Network nodes would also be connected to nodes outside the network, e.g., client and server nodes. Data and control signals from these external nodes would also be carried in separate channels.

Figure 7 –  Dynamic Channel Switching Network

An ideal network would fully connect all subscribers and would transport data without distortion among subscriber nodes at high data rates and with minimal delays. Figure 8 illustrates the operation of an ideal network based on dynamic channel switching. Although clients can communicate with clients and servers with servers, most of the communications is assumed to be between clients and servers, and most of the data is assumed to flow downstream from servers to clients. The flow of information required to control the routing and forwarding within the network is separate from the data flow. In other words, the control plane would be separate from the data plane, as with GMPLS. A single common signaling channel, similar to the ISDN D channel, would be provided to each subscriber (either client or server). This channel would be used to control all the connections for the subscriber, and specifically the capacity of the data channels assigned to each data flow. For example, if a server is about to send data  to a client, then the server would use its signaling channel to request the network to assign channels for the data flow. If sufficient network capacity is available, the network would assign the channels and notify the client via the client’s signaling channel. After the transfer of data is completed, the server would notify the network that the channels are no longer needed and can be released. The network would then release the channels and notify the client.

### Figure 8 –  Operation of a Dynamic Channel Switching Network

Subscribers can access the network via twisted pair, coaxial cable, or fiber optic lines, or by wireless (radio) links. Network nodes would be interconnected by fiber optic cables. The network would support a hierarchy of data channels as illustrated by Figure 9. Different fibers within a cable separate signals spatially and as be viewed as space channels. Multiple signals at different wavelengths can be transported within a fiber so that each space channel can be viewed as containing multiple wavelength channels. Similarly, the signal at each wavelength can be divided into multiple repetitive time intervals, which can be viewed as time slot channels.

### Figure 9 – Hierarchy of Network Channels

The Synchronous Optical Network (SONET) and the Synchronous Digital Hierarchy (SDH) standards define frame structures and data rates for the time-slotted signals used in the proposed network.[15] SONET/SDH frames have a repetition interval of 125 microseconds and data rates that are multiples of 51.84 Mb/s, the fundamental SONET data rate. The fundamental SONET frame, the Synchronous Transport Signal (STS)-1, has a payload that contains approximately 800 bytes and supports a data rate of approximately 50 Mb/s. Each byte within the payload can be viewed as corresponding to a digital signal level 0 (DS0) channel, which can support a data rate of  64 Kb/s. Higher order SONET frames can be formed by byte interleaving of fundamental frames. For example, the STS-3 frame, which is equivalent to the fundamental SDH frame, can be formed by byte interleaving of three STS-1 frames. Higher order SONET/SDH frames are equivalent to a large number of DS0 time slots. The payload of SONET/SDH frames can be segmented into virtual tributaries. For example, an STS-1 frame can be segmented into 28 VT1.5 virtual tributaries, each containing 24 DS0 equivalents.

Other implementations of time based channels within a wavelength channel are possible. For example, a wavelength channel can be divided into Gigabit Ethernet (GigE) frames. In this case, the width  of a frame is variable, but unlike the case with SONET/SDH, the frame cannot be further subdivided.

The space/wavelength/time channels described above would be assigned to connections through the network and dynamically switched based on the current data transport needs of the connections, hence the name, dynamic channel switching. In response to a connection request from a subscriber (a client or a server),  the network would determine a suitable route through the network and assign channels along this route. These channels could be assigned indefinitely or for a specified period of time. Assigning a fixed capacity for an indefinite period of time is equivalent to circuit switching operation. Assigning a set of channels for one or two SONET frames is equivalent to packet switching operation. The establishment of connections and the assignment and switching of channels would be controlled by signals sent over a set of dedicated channels, referred to as the control plane. Switches within the network would establish switching patterns based on information received via the control plane.

After the switching patterns are set up, data would be switched based on its location within space/wavelength/time channels. Control signals would indicate to the switch that data in a particular channel belongs to a particular data flow. Time slots (or virtual tributaries) within a space-wavelength channel would be assigned to a particular connection for a certain period of time. Alternately, a number of successive Ethernet frames within a space-wavelength channel would be assigned to a particular connection. In the language of GMPLS, the fiber, wavelength, and time slot(s) containing the data would be considered to be “implicit labels”. Consequently, the forwarding of data by network switches can be similar to the forwarding of packets using label switching.

For voice operation (typically client to client), a subscriber would use the signaling channel to request a connection to another subscriber. The network would provide a single full duplex DS0 channel (or two simplex DS0 channels) through the entire network for the duration of the call, which is similar to the operation of current telephone networks. With this approach, the utilization of the DS0 channel in each direction would be approximately 45%, reflecting the fact that each speaker in a conversation usually talks less than half the time.[16] However, a single DS0 represents an insignificant percentage of the overall network capacity, and consequently, the utilization efficiency for this channel is of little concern. Note that by using a dedicated DS0 channel, voice samples can be transmitted immediately and delays associated with filling a packet with voice samples are avoided.

Typically, files are transferred from servers to clients. These server-to-client transfers could be accomplished as follows. Using the signaling channel, the client would request that the network establish a connection between the client and a server, which could be viewed as two simplex connections, one in each direction. After the connection is established, the server would automatically download a default file(s) to the client, which could be viewed as the server’s “homepage”. The downloaded file(s) would contain links to other files on the server or to other servers. By clicking on a link, the client could request a file from the server. Alternately, the client could request a connection with a second server, which would terminate the connection with the first server. Normally, a single DS0 channel would be assigned to the path (simplex connection) from the client to the server (the uplink). This channel would provide more than enough capacity for file transfer requests while using an insignificant portion of network resources. Channels would be assigned to the path from the server to the client (the downlink) only when needed. When a server was ready to transfer a file, it would request from the network a channel of a certain capacity, or equivalently a certain number of DS0 channels. Based on channel availability, the network may assign a channel(s) with a capacity lower than the requested capacity. After the file transfer is completed, the server would ask the network to release the channel(s). This approach allows the data flow through the network to be controlled and would help to minimize network delays and congestion. There may be a delay between when the channel(s) is (are) assigned and when the file transfer begins, or between  when the transfer is completed and when channel(s) is (are) released. These delays would reduce the utilization efficiency somewhat but would not adversely affect the transferring of files.

To reduce delays an alternate approach to file transfers could be used. With this latter approach, the file transfer would start right after acknowledgement by the first network node of the server’s request for network capacity. The file transfer would continue if sufficient capacity is available along the end-to-end path from the server to the client. However, if insufficient capacity is available, the file transfer would be aborted. Then server would then request a lower capacity set of channels and re-start the file transfer from the beginning. This approach would be effective if most initial server requests for channels could be satisfied by the network.

Audio and video data is generated in continuous streams rather than discrete blocks. For directly encoded digital audio/video, the data rates are fixed, but can be very high. If audio/video compression techniques, such as the Moving Pictures Experts Group (MPEG) techniques, are employed, these rates can be significantly reduced. For example, with MPEG-2 compression, National Standard Television Committee (NTSC) video rates can be reduced from 216 Mb/s to between 3 and 6 Mb/s [17]. However, MPEG (and other) compression techniques introduce variations in the data rate, including both frame-to-frame variations and scene-to-scene variations. With a moderate amount of output buffering the frame-to-frame variations can be significantly reduced. For MPEG-2 compression with buffering, the ratio of the peak data rate to the average data rate would typically be in the range of  2:1 to 3:1, depending on the amount of motion represented in the video content and the particular implementation.[18]

For audio/video-on-demand of pre-recorded audio/video, operation would proceed as follows. Using the signaling channel, the client would request a connection to an audio/video server. The server would then send a menu to the client. The client would select an audio/video channel and request a particular audio/video stream from the server. The server would then request from the network a downstream channel(s) with sufficient capacity to accommodate the selected audio/video stream. In anticipation of scenes containing more motion and requiring a higher data rate, the server would request additional downstream capacity from the network. If  sufficient capacity is available, the requested capacity would be assigned to the downstream connection. As the required data rate for the audio/video stream decreases, the capacity assigned to the connection could be reduced. If the sufficient capacity cannot be provide by the network, then the server would reduce the data rate of the source, e.g., by increasing the size of the MPEG quantization steps, which would result in some degradation in image quality.

With MPEG compression, video data can be segmented into a base layer and an enhancement layer.[19] An MPEG receiver can operate using only the base layer data with the enhancement layer providing improved resolution. This feature can be exploited in the transmission of broadcast (multicast) video or real-time video. The network would need to allocate enough capacity to accommodate the base layer data for all subscribers receiving the video. If sufficient capacity is available, the network would also provide enough channels with sufficient capacity to accommodate the enhancement layer. Otherwise, for some video users, the MPEG receiver would suffer graceful degradation as the image quality would be limited to the quality provided by the base layer. Note that capacity for the enhancement layer could be allocated dynamically based on changing video data rate requirements.

The primary advantage of dynamic channel switching is that it provides a means for transporting data streams without distortion while simultaneously achieving a high channel utilization efficiency for bursty data. Thus, the proposed networking approach combines the advantages of both circuit switching and packet switching. Specifically, the advantages of dynamic channel switching include:

- Elimination of errors caused by lost, late, or out of order packets;

- Efficient utilization of network capacity;

- Reduced computational burden, as a result of relaxed latency and throughput requirements;

- Reduced overhead with the elimination of packet headers and trailers;

- Reduced delays and congestion, with delays associated with filling of packets eliminated and the need for queuing at network switches reduced;

- Compatibility with existing networks, particularly legacy telephone networks.

The disadvantage of dynamic channel switching is that it is different than the modes of operation used by current networks. Thus, to achieve the full benefits of dynamic channel switching, existing network and subscriber equipment and network operations and would need to be modified. The next section addresses how this disadvantage can be mitigated.

Backward Compatible Network Operation

The dynamic channel switching approach described in the previous section is compatible with legacy telephone networks. Dynamic channel switching can be viewed as a special case of the circuit switching techniques employed by these legacy networks. The user-to-network interface (UNI) for control signaling would be compatible with the ISDN D channel signaling and network node interface (NNI) signaling protocols could be made to include SS7 as a subset. When interfacing with legacy telephone networks, the speed and flexibility advantages of dynamic channel switching may not be realized. However, the new network would be able to interface transparently with legacy telephone networks, and thus be backward compatible with these networks.

The proposed network can also be made backward compatible with current Internet operation. Clients and servers can be directly interconnected through the proposed network, which would enable the clients and servers to reap the benefits of dynamic channel switching. However, the clients and servers would need to update their equipment to implement dynamic channel switching protocols. Another mode of operation would be for the proposed network to provide conventional circuit switched connections from clients and servers to ISPs, and specifically to Internet routers. This approach would be compatible with legacy clients and servers, but would not provide the benefits of dynamic channel switching.

A third option, which is illustrated by Figure 10, provides most of the benefits of dynamic channel switching, but requires equipment modifications for only a small percentage of the subscribers. With this option, client equipment would remain unchanged. There would be separate simplex connections for the client-to-server and server-to-client data flows. Clients would still establish circuit switched connections to an ISP and would send IP packets through the Internet to a server. However, data flow in the reverse direction would be able to bypass the Internet. Server equipment would be upgraded to include dynamic channel switching protocols. Servers would continue to send data using IP packets, but servers would be able to use the control plane to establish data flows through the network and identify which packets belong to a particular data flow. A server would signal the network when it was ready to send data, and the network would set up channels to accommodate this data flow prior to receiving the data. Network elements would route packets based on their location within the channel structure. This would make IP superfluous from downstream transmission. Clients would receive IP packets as they currently do and would be unaware that the Internet had been bypassed. Thus, this approach could provide the benefits of dynamic channel switching for downstream data, which represents the bulk of the data flow, without impacting client equipment.

For voice over IP operation, a single DS0 channel would be established in each direction from a subscriber through the proposed network to an ISP supporting voice over IP. IP packets containing voice samples would be sent through the Internet and on to the subscriber at the other end of the conversation. Subscribers on both ends would transmit and receive voice samples encapsulated in IP packets. With this mode of operation, the proposed network would be compatible with voice over IP operation, but would not provide any significant benefits relative to current networks. However, since only a single DS0 channel (in each direction) is required, voice over IP operation is not expected to have a significant impact on network resources.

### Figure 10 – Backward Compatible Network

For file transfers, TCP connections would be established between clients and servers. To support TCP connections, each client and server would need to have one or more circuit switched connections through the proposed network to an ISP. A client would send a file transfer request to a server through the Internet. A server that was upgraded to support dynamic channel switching would request  downstream channels from the network prior to sending the requested file to the client. The file would be broken up into IP packets and transferred through the proposed network bypassing the Internet. The transfer of data from the server to the client would follow either of the two alternate approaches described in the previous section. The client would receive IP packets in the normal way and would be unaware that the packets did not traverse the Internet. After the file transfer was completed, channels would be released back to the network. This approach would speed up the actual transferring of files without tying up network resources for longer than needed.

Audio/video operation would be similar in some respects to file transfers, except that video data is generated continuously rather that a discrete times. The client would request that the network establish a connection with a video server and send IP packets to the server via the Internet. The server would then provide the client with a menu of available audio/video streams. After the client selected an audio/video stream and sent a request for this stream, the server would request from the network a  downstream channel(s) with sufficient capacity and start transferring the audio/video stream. The methods for transfer of streaming audio/video from the server to the client would be similar methods described in the previous section for transferring pre-recorded video and broadcast/real-time video. In both cases, audio/video data in IP packets would be sent through the proposed network bypassing the Internet. Again, the client would receive the packets in the normal way and would be unaware the packets did not traverse the Internet.

In summary, the operation of the proposed network can be made backward compatible with Internet operation. Most of the benefits of dynamic channels switching, which were listed in the previous section, can be achieved if only server equipment is modified to implement dynamic channel switching protocols. Client equipment does not need to be modified for clients to share in the benefits. Since clients make up the vast majority of network subscribers, most of the benefits of dynamic channel switching can be provided without incurring the cost of upgrading most subscribers.

## Summary

Telephone networks are based on circuit switching while the Internet is based on packet switching. Typically, both clients and servers access the Internet via circuit switched connections through the telephone networks. With circuit switching, a physical layer connection is established through the network and the data stream remains undistorted as it traverses the network. However, circuit switching requires dedicated channels, and if the data source is bursty, this results in an inefficient utilization of transmission resources. With packet switching, transmission resources are dynamically shared, which makes packet switching efficient for transmitting bursty data. However, packet switching requires more complex data processing, and it introduces variable delays, which produce distortions in the data stream.

A networking approach was presented that combines the advantages of circuit switching and packet switching in an integrated network. With this approach, control signals are separated from the data and transmitted separately in a common control channel. Control and signaling operation can be viewed as a logical extension of GMPLS operation. A variable number of data channels are dynamically assigned to each connection through the network. Channels are quickly switched in to accommodate the data flow through a connection and switched out when they are no longer needed. With this dynamic channel switching approach, a continuous data stream is not altered as it transverses the network and data channels are efficiently utilized even if the data source is bursty.

The proposed network is compatible with legacy networks. To take full advantage of this network’s capabilities, client and server equipment must be upgraded to support dynamic channel switching. However, most of the benefit of the proposed approach can be achieved if only server equipment, but not client equipment, is upgraded. In this case, data transport from servers to clients, which represents the bulk of the data flow, will be enhanced, but client equipment, which represents the bulk of subscriber equipment, will not be affected. Consequently, the proposed approach provides a mechanism for facilitating the convergence of circuit switching and packet switching.

References

[1] S. Pandhi, “The Universal Data Connection”, IEEE Spectrum, July 1987.

[2] Y. Lin, et al, “Fiber-Based Local Access Network Architectures”, IEEE Communications

Magazine, Vol. 27, No. 10, October 1989.

[3] E. Rosen, et al, “Multiprotocol Label Switching Architecture”, IETF Network Working Group,

RFC 3031, January 2001.

[4] A. Banerjee, et al, “Generalized Multiprotocol Label Switching: An Overview of

Routing and Management Functions”, IEEE Communications Magazine, Vol. 39,

No. 1, January 2001.

[5] G. Hawley, “Historical Perspectives on the U.S. Telephone Loop”, IEEE

Communications Magazine, Vol. 29, No. 3, March 1991.

[6] J. Martin, Telecommunications and the Computer, Englewood Cliffs, New Jersey:

Prentice Hall, 1976, pp. 419-420.

[7] A. Modarressi and R. Skoug, “Signaling System No. 7: A Tutorial”, IEEE

Communications Magazine, Vol. 28, No. 7, July 1990.

[8] T. Bartee, editor, Digital Communications, Indianapolis, Indiana: Howard W.

Sams & Co., 1986, pp. 116-124.

[9] A. Day, “International Standardization of BISDN”, IEEE LTS – The Magazine of Lightwave

Telecommunication Systems, Vol. 2, No.3, August 1991.

[10] W. Stallings, Data and Computer Communications (sixth edition), Upper Saddle

River, New Jersey: Prentice Hall, 2000, pp. 536-537.

[11] W. Stallings, op. cit., pp. 573-582.

[12] R. Braden, ed., “Resource ReSerVation Protocol (RSVP) - Version 1 Functional

Specification”, IETF Network Working Group, RFC 2205, September 1997.

[13] G. Swallow, et al, “RSVP-TE: Extensions to RSVP for LSP Tunnels”, IETF Network

Working Group draft, August 2001.

[14] W. Stallings, Local Networks, New York: Macmillan Publishing Company, 1984,

pp. 180-182.

[15] C. Omidyar and A. Aldridge, “Introduction to SDH/SONET”, IEEE

Communications Magazine, Vol. 31, No. 9, September 1993.

[16] J. Martin, op. cit., pg. 241.

[17] B. Lyles and D. Swinehart, “The Emerging Gigabit Environment and the Role of Local

ATM,” IEEE Communications Magazine, Vol. 30, No. 4, April 1992.

[18] P. Pancha and M. El Zarki, “MPEG Coding for Variable Bit Rate Video

Transmission”, IEEE Communications Magazine, Vol. 32, No. 5, May 1994.

[19] T. Sikora, “MPEG-1 and MPEG-2 Digital Video Coding Standards”,
        http://www am.hhi.de/mpeg-video/papers/sikora/mpeg1_2/mpeg1_2.htm,

downloaded 3 October 1997.