Chemical Safety

Working with potentially hazardous chemicals is an everyday occurrence in a laboratory setting. Hazardous situations can occur if employees are not educated in general chemical safety, toxicological information, and procedures for handling and storage for the chemicals they are using. This section of the laboratory manual addresses these educational components and spells out specific protocols to minimize hazardous chemical exposures.

A. Modes of Entry

There are four major modes of entry to chemicals: inhalation, skin absorption, injection, and ingestion. Inhalation and skin absorption are the predominant occupational exposures you may expect to encounter in the laboratory and will be discussed in some detail. Accidental injection of chemicals can be eliminated by good laboratory safety practices. Accidental ingestion of chemicals can be eliminated by a combination of good laboratory and hygienic practices such as washing hands and prohibiting foods, drinks, cosmetics, and tobacco products in the laboratory workplace. All potential exposures, i.e., inhalation, skin absorption, injection, and ingestion, are discussed in the Material Safety Data Sheets available for each chemical or product. The hundreds of chemicals which employees are routinely exposed to during the course of their work in the laboratory can be divided into three main types: volatile solvents, corrosives, and toxic solids. The particular hazards associated with exposure to these materials, and ways to avoid them, are discussed in detail below.

B. Basic Chemical Classifications

1. Volatile Solvents

Organic solvents are perhaps the most ubiquitous chemicals found in the laboratory setting. The potential chronic health effects of some of these materials warrant special attention as one is likely to be exposed to more solvents than any other type of chemical. For safety purposes, these chemicals are generally subdivided into two categories: chlorinated and non-chlorinated. This is done mainly because the chlorinated solvents are, in general, not flammable while non-chlorinated solvents are often flammable. It should be kept in mind, however, that the chlorinated solvents do decompose when burned. This results in high concentrations of toxic vapors, such as phosgene and hydrogen chloride.

Keeping in mind the difference in flammability between these two classes of solvents, we can discuss the health effects common to both classes. The primary route of exposure to these materials is through inhalation. In general, high concentrations of the vapor, when inhaled, produce drowsiness, dizziness and headaches. This can occur quite quickly, since chemical vapors are rapidly absorbed. Most of the solvents will also act as upper respiratory and/or eye irritants. One physical property common to most solvents is odor. Unfortunately, the odor of a solvent offers little in the way of determining whether or not the environment is immediately hazardous. Solvent odor thresholds vary widely and acclimation or odor fatigue is often rapid. Odor is also not generally indicative of the degree of hazard that the material presents. Butyl mercaptan has such an extremely disagreeable odor that one cannot tolerate a concentration necessary to be injurious. Chloroform, however, has a sweet odor to many people and tolerance levels can far exceed safe levels.

Chronic effects of solvent exposure vary widely. Of most concern is the potential for lung, liver, and kidney damage posed by some solvents. This, in general, applies to solvents which are not water soluble. Examples of these solvents would be benzene, toluene, xylene, chloroform, carbon tetrachloride, and trichloroethylene. Instances of chronic disease caused by occupational exposure to these solvents have been documented. However, it must be kept in mind that everyone reacts differently and individual susceptibilities are quite variable.

Skin absorption is an additional mode of entry for which an exposure to a solvent may occur. Most commonly, solvents act to de-fat the skin. This will cause drying and cracking of the skin, and may lead to chronic dermatitis with prolonged and repeated exposure. Some solvents can also act as corrosives. Most amines and phenols act in this manner.

In addition, many of the solvents (dimethyl sulfoxide and dimethyl formamide, for example) will penetrate the skin and be absorbed into the body. In this case, the effects of exposure will be analogous to inhalation exposure. Carbon disulfide, n-butyl alcohol, and phenol are common solvents which can penetrate intact skin. For those solvents, there will be a notation of skin exposure noted on the Material Safety Data Sheet. Most skin contact with solvents can be avoided by wearing gloves suitable for that chemical. It is important that the glove be resistant to the material being handled. Using the wrong glove can give a false sense of security and overexposure via the skin may result. If a solvent penetrates the glove, a prolonged contact will result due to slowed evaporation rates. Rubber and neoprene gloves can be classed as good general purpose gloves, but a chemical resistance chart and the MSDS should always be consulted.

Direct liquid contact by solvents in the eyes can be very serious. The victim could easily panic. Get them to the eye wash immediately and flush the eyes for at least 15 to 30 minutes. Medical assistance should also be summoned.

In summary, volatile solvents can pose inhalation, skin, and ingestion hazards. Some of the solvents may also be flammable, which could cause fire and/or explosion hazards. Whenever possible, use volatile solvents in a properly operating fume hood to eliminate inhalation hazards, use correct skin and eye protection and use good laboratory and hygienic technique to eliminate any possible ingestion of volatile solvents.

2. Acid and Bases

Common to all acids and bases is their corrosive action on human tissues. Minor exposures are generally reversible, although often painful for a short period of time. The reversibility of the effects of acid or base exposure will depend on three factors: the duration of exposure, concentration of the material, and the first aid methods used. Exposure can occur through skin absorption or inhalation. With inhalation exposure, remove the victim from the area (try to keep the victim from breathing too deeply, as this may exacerbate the effects) and summon medical help.

Skin contact is the most common route of exposure. Here the concentration and type of acid are the most important factors. In concentrated forms, all types of corrosives may cause severe penetrating burns. Dilute solutions do not have the same warning properties as concentrated forms, so guard against exposure. One should be particularly careful with hydrofluoric acid.

Neoprene gloves provide the best protection from skin exposure to both acids and bases, but in all cases, follow the recommendations in the MSDS. When using or dispensing concentrated acids or bases, a lab coat or apron and a full face shield is required.

If there is skin or eye contact with acids or bases, make sure to flush the area with water for 15 to 30 minutes and summon medical assistance.

3. Toxic Solids

Many of the chemicals used in the laboratory that are solid and toxic are used in solution, so skin absorption can be of a concern. This is particularly true when a substance is dissolved in a solvent which can penetrate the skin. Also, an oxidizing material dissolved in water can act directly on the skin causing irritation where the solid alone would be relatively less irritating. It is therefore important that proper personal protective equipment be worn.

In the solid form, the greatest risk of exposure is through inhalation. This risk can be lessened by wearing the appropriate respirator and/or working in a fume hood.

C. Incompatible Chemicals

Certain hazardous chemicals cannot be mixed or stored safely with other chemicals due to potentially severe or extremely toxic reactions taking place. For example, keep oxidizing agents separated from reducing agents, initiators separated from monomers, and acids separated from alkalis, etc. The chemical label and Material Safety Data Sheet will contain information on incompatibilities. A list of incompatible chemicals is included in Appendix B.

D. Chemical Stability

Stability refers to the susceptibility of the chemical to decomposition. Ethers, liquid paraffins, and olefins can form peroxides on exposure to air and light. Since these chemicals are packaged in an air atmosphere, peroxides can form even though the containers have remained sealed. Some inorganic chemicals also are unstable.

Unless inhibitor was added by the manufacturer, closed containers of ethers shall be discarded after one year. Appropriate use of peroxide inhibitors is suggested.

Examples of potential peroxide forming materials are included in Appendix A.

E. Procurement of Chemicals

The achievement of safe handling, use, and disposal of hazardous substances begins with the persons who requisition such substances and those who approve their purchase orders. These persons must be aware of the potential hazards of the substances being ordered, know whether or not adequate facilities and trained personnel are available to handle such substances, and should ensure that a safe disposal route exists.

No container or cylinder should be accepted that does not have an identifying label. For chemicals, it is desirable that this label correspond to ANSI Z129.1, which requires, at a minimum, the following components:
1. Identification of contents of container;
2. Signal word and summary description of any hazard(s);
3. Precautionary information - what to do to minimize hazard or prevent an accident from happening;
4. First aid in case of exposure;
5. Spill and cleanup procedures; and
6. If appropriate, special instructions to physicians. Every effort should be made to ensure that this label remains on the container and legible.

F. Spill Prevention

A hazardous chemical spill means that an uncontrolled release of a hazardous chemical has occurred. The release may involve a gas, liquid, or solid, and usually requires some action be taken to control the point of release or the spread of the chemical. A chemical is hazardous if it possesses a physical or health threat to humans, the environment, or property. More specifically, a substance is considered hazardous when:
a. It is flammable, explosive, or reactive;
b. It generates harmful vapor or dust;
c. It is a carcinogen;
d. It is a corrosive and attacks skin, clothing, equipment, or facilities;
e. It is poisonous by ingestion, inhalation or absorption.

Spills involving hazardous materials will require different tactics depending on the magnitude of the spill, the material's toxicity, reactivity, and flammability, routes of entry of the material into the body, and the promptness with which the spill can be safely managed.

Many spills can be prevented or controlled by careful planning, use of trays, and absorbent paper. (Remember, hoods don't prevent or control spills; they just relocate them!) Proper techniques for transporting hazardous chemicals and proper storage techniques may help prevent spills.

G. Handling and Transportation of Chemicals

Many laboratory accidents occur through the simple operation of carrying chemicals from one place to another or transferring them from one container to another. The chemicals used in a laboratory are often corrosive, toxic, or flammable and any accident involving these has the potential for personal injury. Therefore, it is good practice to assume that all chemicals are potentially hazardous.
1. When large bottles of acids, solvents, or other liquids are transported within the laboratory without a cart, only one bottle should be carried at a time. The bottle should be carried with both hands, one on the neck of the bottle and the other underneath. Avoid the temptation to hook a finger through the glass ring on top of the bottle, allowing it to dangle while being transported. Never carry or attempt to pick up a bottle by the cap.
2. When transporting bottles within the laboratory, a wheeled cart may be used. Carts should be stable under load and have wheels large enough to negotiate uneven surfaces (such as expansion joints and floor drain depressions) without tipping or stopping suddenly. Do not place the bottles near the edge of the cart, nor should they be touching each other or other glassware during transport. Be cautious rolling the cart over door sills or other possible obstructions. Incompatible chemicals should not be transported on the same cart.
3. Freight-only elevators should be used, if possible, when transporting chemicals, to avoid exposure to persons on passenger elevators.
4. Special padded or rubber bottle carriers, pails, or carts should be used to prevent breakage by accidental striking against walls or floor, and to contain the material if breakage does occur.
5. Large quantities of concentrated mineral acids, e.g., sulfuric, nitric and hydrochloric acids, shall be kept in storage rooms, in cabinets for corrosive substances, or chemical transfer rooms. Bottles of concentrated acids must be carried from the aforementioned areas in an approved acid bottle carrier.
6. Organic solvents shall also be stored in specialized flammable storage areas. These solvents shall be carried from storage areas in special rubber carriers. Organic solvents can present fire hazards as well as inhalation hazards.

H. Chemical Storage

The principle concerns in achieving proper storage are to maximize employee safety with regard to chemical compatibility, spill control, fire/explosion control, to provide security, identification, and provide a "user friendly" system with respect to point-of-use.

1. Every chemical in the laboratory should have a definite storage place and should be returned to that location after each use.

2. Storage must conform to compatibility restrictions. Typically, solvents, acids, bases, reactives, oxidizers, and toxins will be stored separately. Separation basically refers to physical separation of containers and isolation of potential spills and releases with the goal of preventing chemical reactions. Ideally, separate cabinets or isolated areas within a central storage area should be utilized for segregated storage of incompatibles.

3. Adequate containment for spills and accidental releases shall be provided.

4. Hazardous chemicals should never be stored on the floor. Containers should be kept on low shelves or in cabinets. The shelves should have a lip on the forward edge to prevent bottles from slipping off. Chemicals tend to "creep" toward and over the edge of a shelf. Shelving units should be securely fastened to the wall or floors. Shelves should not be overloaded.

5. Utilize a compatible/suitable container for experiments, stored chemicals and collected wastes. In instances of corrosive wastes or halogenated solvents, the use of metal containers is often unsuitable, even if the solvents were originally shipped in metal containers. In these instances, plastic carboys (high density polyethylene) or lined metal containers may be more suitable.

6. There shall be constant vigilance for any sign of chemical leakage. Containers storing chemical waste must be inspected weekly for any sign of chemical leakage. Containers of all types should be free of rust and deformation.

7. Caps and covers for containers shall be securely in place whenever the container is not in immediate use.

8. Storage shall be physically secure.

9. NFPA labeling shall appear on cabinets and room doors at approximately waist level or lower to allow adequate visualization in dense smoke conditions.

10. All containers used for storage (even short term) shall be labelled. At a minimum, all containers must be labeled with regard to content and general hazard.

11. Flammable liquids in quantities greater than one liter should be kept in metal safety cans designed for such storage. The cans should be used only as recommended by the manufacturer, including the following safety practices:
a. Never disable the spring-loaded closure.
b. Always keep flame-arrestor screen in place; replace if punctured or damaged.

12. Flammable liquids shall not be stored in your laboratory unit in amounts greater than the limits for flammable liquid storage.

13. Metal drums used for storage and dispensing of flammable chemicals shall be properly grounded. Ground cables shall be available and utilized in any lab using metal storage containers for flammable liquid storage.

14. Chemicals should be stored as close as feasible to the point of use in order to maximize efficiency and minimize transport distance. Chemical storage should be limited only to areas in which the particular chemical is used. Storage locations must be identified on an emergency floor plan posted in each work area and should be equipped with a fire extinguisher, spill kit, eye wash, first aid kit, and telephone or other communication system to allow for adequate emergency notification.

15. Small quantities of chemicals can be held at individual work stations if this quantity is to be promptly used in a test and does not compromise acceptable ambient organic vapor levels or procedures for spill control and fire safety. These containers must be properly labeled.

16. Only limited quantities of chemicals and solvents should be stored in the laboratory. Large drums or multiple bottles of chemicals should be stored in a centralized chemical storage area.

17. Out-of-date chemicals shall be disposed of on a periodic basis to reduce overall hazard potential and minimize inventory tracking and updating.

I. Storage of Waste Chemicals

Waste Chemicals shall be stored in the same manner and using the same procedures as other chemicals. It may be advantageous to further segregate chemical waste. A typical segregation of waste chemicals would be:

- Acids
- Caustics
- Chlorinated Solvents
- Non-chlorinated Solvents
- Mercury Wastes
- Oxidizing Agents
- PCB Wastes*
- Reactive Chemicals*
- Waste Oil
- Wastes with Heavy Metal Contamination

*Consult with Environmental Health & Safety regarding proper storage and disposal requirements.

These chemicals shall be accumulated in separate containers and need to be isolated from one another to some degree, at least to the extent that spills or leaks would remain isolated from other containers. This is particularly true of acids, bases, and solvents.

J. Hazard Identification

Each material shall be identified with a "Hazard Code" abbreviation to be used as a general description to provide information on handling hazards and responsive action in the event of an accident. Each material shall be identified on the form by the hazard that most closely indicates the greatest hazard presented by the material. The following is a list of hazards and their definitions. The codes are listed on the form.:

Flammable: Any compressed gas, liquid, or any solid material (other than an explosive, heat sensitive or shock sensitive material) that is liable to cause fires through friction, absorption of moisture, spontaneous chemical changes, retained heat from processing, or which can be ignited readily, and when ignited burns so vigorously and persistently as to create a serious transportation hazard. Examples: Acetone, methyl alcohol, dimethylamine, propane, sodium dithionite, nitrocellulose.

Corrosive: Any gas, liquid, or solid that causes destruction of human tissue or a liquid that has a severe corrosion rate on steel or aluminum. Aqueous solutions that have a pH equal to or less than 2 or equal to or greater than 12.5 are corrosive. Other liquids are defined as corrosive if they corrode SAE 1020 steel at a rate greater than 6.35 mm/year at 55oC. Examples: Hydrochloric acid, sulfuric acid, acetic acid, sodium hydroxide.

Oxidizer: A substance such as chlorate, permanganate, inorganic peroxide, nitrocarbonitrate, or a nitrate that yields oxygen readily to stimulate the combustion of organic matter. (Organic peroxides are to be listed under "OTHER", and the hazard listed on the bottom of the form.) Examples: Sodium nitrate, potassium permanganate, manganese dioxide.

Toxic: Substances such as carcinogens, irritants, or poisonous gases, liquids, and solids which are irritating to or affect the health of humans. Examples: Phosgene, phenol, 1-naphthylamine, arsenic compounds, mercury.

Water Reactive: Substances that react violently when in contact with water. They can be either be flammable solids or corrosives. (Mark the label with both Water Reactive and flammable or corrosive). Note: In many cases the water reactive material can be chemically treated, thereby removing the reaction characteristic. The material may still contain other hazardous constituents. Please contact OSU HAZMAT for further information and guidance. Examples: Lithium, potassium or sodium metal, antimony pentachloride, acetic anhydride, calcium carbide.

Carcinogens: Any substance that causes the development of cancerous growths in living tissue, either those that are known to induce cancer in man or animals or experimental carcinogens that have been found to cause cancer in animals under experimental conditions. Examples: 1-Naphthylamine, benzidine, dimethyl sulfate. Teratogens/Mutagens: Teratogens are agents that cause growth abnormalities in embryos, genetic modifications in cells, etc. Mutagens are substances that are able to induce mutations in DNA and in living cells. Examples: Diethylstilbestrol (DES), diethyl sulfate.

Other: Special hazards such shock or heat sensitive, organic peroxides, pyrophorics (reacts with air), peroxide formers. Examples: Picric acid, urea nitrate, 2,4-dinitrophenylhydrazine, benzoyl peroxide, phosphorus, isopropyl ether.

K. Hazardous Materials Surplus Program

A limited number of chemicals, including (but not limited to) the following will NOT be placed in surplus due to their potentially dangerous nature (the following list of chemicals should be rendered inactive by the user prior to disposal):
- Alkyl boranes
- Aluminum Alkynes
- Ammonium Nitrate
- Benzoyl Peroxide
- Calcium Carbide
- Chromic Acid
- Cyanides
- Ethers
- Grignard Reagents
- Hydrogen Peroxide
- Iron Sulfide
- Metal Alkyls
- Metal Hydrides
- Peracetic Acid Solution
- Peroxide Forming Compounds
- Picric Acid
- Sulfides
- Water reactive metals (Lithium, Potassium, Sodium, Cesium)
(NOTE: Due to the explosive characteristics inherent to the addition of a carbon source to Ammonium Nitrate, Ammonium Nitrate shall not be placed in surplus.)

Information extracted by Marcus Chua