Tutorial 2
 (Power, Series Circuits, Circuit Ground)

- Deduce from first principle an expression for VR1 in terms of Vs, R1 and R2 in figure 1.

VR1

Vs

VR2

R1

R2

- Figure 1

- Hence or otherwise, determine the voltages VR1 , VR2 , the resistors R1 , R2 and the current I in figure 1:

| Vs (V) | R1 () | R2 () | VR1 (V) | VR2 (V) | I (mA) |
| --- | --- | --- | --- | --- | --- |
| 10 | 100 | 100 |  |  |  |
|  |  |  | 5 | 5 | 200 |
| 20 |  | 100 |  | 5 |  |
|  | 100 |  |  | 4 | 20 |
|  | 50 |  | 5 | 5 |  |

- Find the equivalent resistance for each of the series combination:

1 

4 

6 

2 

3 

Figure Q3-1

Figure Q3-2

- For each case,

  - Using the equivalent series resistance, find the voltage across the series combination when I = 1 A;

  - Find the voltage drop across each resistor when I = 1 A and verify that the sum of the voltage drops adds up to the answers in (a).

4. To measure the current in a resistor, an ammeter should be connected _____________.

5.	Refer to the circuit below, determine :

(a) the current in the circuit, and

(b) the voltage across the points A and B.

6.	A mains energised ( assume voltage = 220 V )Christmas light chain has 44 light bulbs connected in series. If the maximum allowable current is 10 mA, what should be the filament resistance of individual bulb ? What should be the power rating of the bulbs ?

27 V

5V

+ V -

2.2K

0.56K

1.2K

I

9V

7.

For the circuit above, determine:

i. the current I; and

ii. the voltage V.

- In telecommunications, power is a precious resource. The power level a signal determines how far it can travel, and also how fast the data rate can be without errors. 

(i) Compare the relative amount of power needed in transmitting a signal between (a) a home cordless phone and its base station;. (b) your GSM/PCS/CDMA mobile phone and the mobile base station , (c) a satellite and its earth station.

(ii) Assume copper cable is the transmission media, comment on which signal is capable of transmitting a longer distance and can have a higher data rate: (a) Signal A power = 50 mW  (b) Signal B power = 200 mW.

(iii) Comment on the validity of this statement” “As long as we increase the power of a signal, we can make the data rate as fast as we like”.